Routing table

Table of contents

Display	2

Cisco routing table analyses	2

Cisco routing algorithm	2

Examples	3

# Display

To display a routing table on a Cisco router:

R1 #show ip route

To display a routing table on a Windows device:

C:\Users\user>route print

To display a routing table on a Linux device:

user@machine$ip route

# Cisco routing table analyses

D 192.168.2.128/26 [90/2345678] via 192.168.2.254, 01:10:20, gigabit0

S 192.168.2.128/28 [1/0] via 192.168.2.150

R 192.168.2.128/27 [120/3] via 192.168.2.1, 00:00:15, gigabit1

O 192.168.2.128/29 [110/5000] via 192.168.2.253, 00:20:10, gigabit2

S* 0.0.0.0/0 [1/0] via 192.168.2.100

ADMIN DISTANCE: it’s a distance that the admin put

METRIC: it’s the total distance (how far it is?)

TIME: it’s the last time it hears about the destination

INTERFACE: it’s the interface to use to the destination

NEXT HOP: it’s a router that is closer to the destination than its

CANDIDATE DEFAULT: it’s the default route to use

| D | EIGRP |
| --- | --- |
| S | Static |
| R | RIP |
| O | OSPF |

If an entry in the routing table doesn’t have an interface, the router makes another pass through the routing table, and so on.

# Cisco routing algorithm

- Longest prefix mask (/32 then /31 then /30 then … then /0)

- Lowest ADMIN DISTANCE

- Lowest METRIC

The router starts with the longest prefix mask and moves to the lowest. Then the router looks the ADMIN DISTANCE then THE METRIC if the ADMIN DISTANCE is equals.

# Examples

- D 192.168.2.128/26 [90/2345678] via 192.168.2.254, 01:10:20, gigabit0

- S 192.168.2.128/28 [1/0] via 192.168.2.150

- R 192.168.2.128/27 [120/3] via 192.168.2.1, 00:00:15, gigabit1

- O 192.168.2.128/29 [110/5000] via 192.168.2.253, 00:20:10, gigabit2

- S* 0.0.0.0/0 [1/0] via 192.168.2.100

Imagine you’re R1 and you receive a packet to 192.168.2.145. Which route to the router will choose? 3

Step 1: Check matching networks

- /29 → range: 192.168.2.128 – 192.168.2.135 (not included)

- /28 → range: 192.168.2.128 – 192.168.2.143 (not included)

- /27 → range: 192.168.2.128 – 192.168.2.159 (included)

- /26 → range: 192.168.2.128 – 192.168.2.191 (included)

Step 2: Choose longest prefix

- /27 is more specific than /26

Imagine you’re R1 and you receive a packet to 192.168.2.199. Which route to the router will choose? 5

Step 1: Check matching networks

- /26 → range: 192.168.2.128 – 192.168.2.191 (not included)

- /27, /28, /29 → all (not included)

Step 2: No match found → use default route

- Route 5 (S 0.0.0.0/0)* is selected