AAA

Authentication, Authorization & Accounting

Table of contents

On the devices	2

Authentication	2

Authorization	2

Privilege access	2

SSH access	3

Authentication proxy access	4

802.1X Ethernet access	5

On the authenticator device	5

On the Windows device	6

On the ISE server	8

Add network devices	8

Add identities	9

Add policies	11

Create conditions	11

# On the devices

Debug:

R1 #debug radius

R1 #show ip auth-proxy cache

Enable AAA on a router:

R1 (config) #aaa new-model

Specify a RADIUS server:

R1 (config) #radius server MYRADIUS

R1 (config-radius-server) #address ipv4 192.168.2.1

R1 (config-radius-server) #key SECRETKEY

Or the old version:

R1 (config) #radius-server host 192.168.2.205 key SECRETKEY

## Authentication

R1 (config) #aaa authentication SERVICE LISTNAME METHOD1 METHOD2 …

- login: all the possibilities to connect to the router (SSH, Telnet, console, HTTP, auxiliary…)

- ppp: like the service providers

- dot1x:

- local: use the user in local (username admin privilege 15 password cisco)

- none: no verification

- group radius: use the RADIUS server

- default: use it by default

## Authorization

R1 (config) #aaa authorization SERVICE LISTNAME METHOD1 METHOD2 …

- exec: the privilege level of the administrator on the router (0-15)

- auth-proxy: the ACL for the user of the network

- none: no verification

- group radius: use the RADIUS server

## Privilege access

Put a privilege level to access a command:

>  #  (config)  (config-if)  shutdown (level 5):

R1 (config) #privilege exec level 5 configure

R1 (config) #privilege configure level 5 interface

R1 (config) #privilege interface level 5 shutdown

## SSH access

Enable AAA on the router:

R1 (config) #aaa new-model

Specify the RADIUS server:

R1 (config) #radius-server host radius-server-ip key shared-key

Protect the console port:

R1 (config) #username cisco password cisco

R1 (config) #aaa authentication login CONSOLENOAAA local (Warning! Create user before)

R1 (config) #aaa authorization exec IWANTPRIV15 none

R1 (config) #line console 0

R1 (config-line) #login authentication CONSOLENOAAA

R1 (config-line) #authorization exec IWANTPRIV15

Others with the RADIUS server:

R1 (config) #aaa authentication login default group radius

R1 (config) #aaa authorization exec default group radius

Configure SSH:

R1(config) #ip domain-name test.lan

R1 (config) #crypto key generate rsa modulus 2048

R1 (config) #line vty 0 4

R1 (config-line) #transport input ssh

R1 (config-line) #login authentication default

R1 (config-line) #authorization exec default

## Authentication proxy access

Enable AAA on the router:

R1 (config) #aaa new-model

Specify the RADIUS server:

R1 (config) #radius-server host radius-server-ip key shared-key

Protect the console port:

R1 (config) #aaa authentication login CONSOLENOAAA local (Warning! Create user before)

R1 (config) #line console 0

R1 (config-line) #login authentication CONSOLENOAAA

Change authorization of the console port:

R1 (config) #aaa authorization exec IWANTPRIV15 none

R1 (config) #line console 0

R1 (config-line) #authorization exec IWANTPRIV15

Refer RADIUS for authentication & authorization:

R1 (config) #aaa authentication login default group radius
R1 (config) #aaa authorization auth-proxy default group radius

Enable the web server on the router:

R1 (config) #ip http server

Enable AAA for the web server:

R1 (config) #ip http authentication aaa

Create ACL to permit the user access the web server:

R1 (config) #access-list 100 permit tcp 192.168.1.0 0.0.0.255 host 192.168.1.254 eq 80

R1 (config) #access-list 100 deny ip 192.168.1.0 0.0.0.255 any

Configure the proxy:

R1 (config) #ip auth-proxy name HTTPPROXY http list 100

Apply the proxy and the ACL on the interface facing the user:

R1 (config) #int g0/0/0

R1 (config-if) #ip auth-proxy HTTPPROXY

R1 (config-if) #ip access-group 100 in

# 802.1X Ethernet access

## On the authenticator device

On the authenticator switch, configure AAA:

SW1 (config) #aaa new-model

SW1 (config) #radius-server host radius-server-ip key shared-key

SW1 (config) #aaa authentication dot1x default group radius

SW1 (config) #aaa authorization network default group radius

Enable the routing on the switch:

SW1 (config) #ip-routing

Enable the 802.1X protocol on the switch:

SW1 (config) #dot1x system-auth-control

Create some VLANs (X times if X VLANs):

SW1 (config) #int vlan 2

SW1 (config-if) #ip address 192.168.2.254 255.255.255.0

Configure DHCP (X pools for X groups):

SW1 (config) #ip dhcp excluded-address 192.168.2.254 192.168.2.254

SW1 (config) #ip dhcp pool GROUP2

SW1 (dhcp-config) #network 192.168.2.0 255.255.255.0

SW1 (dhcp-config) #default-router 192.168.2.254

SW1 (config) #service dhcp

On each interface facing end user:

SW1 (config-if) #spanning-tree portfast

SW1 (config-if) #switchport mode access

On each interface facing end user, enable 802.1X verifications on the interface:

SW1 (config-if) #dot1x port-control auto

Or

SW1 (config-if) #dot1x pae authenticator

SW1 (config-if) #authentication port-control auto

If 2 MAC addresses are on the same interface, on each interface facing end user:

SW1 (config-if) #authentication host-mode multi-host

Or

SW1 (config-if) #dot1x host-mode multi-host

## On the Windows device

Start the “Wired autoconfiguration” service. (Configuration automatique du réseau câblé in french):

- Win+R services.msc

- Right-click  start

Import the ISE server certificate:

- Download the certificate from your ISE server

- Convert it to a .DER format

- Double-click on the file and choose “install as a trusted root certificate”

Configure the network card:

- There will be a new tab on your network card Win+R ncpa.cpl

- Choose PEAP

- Click on activate IEEE 802.1X authentication

- Click on the “parameters” button

- Check “Verify server identity…”

- Check the ISE server certificate

- Click on the “additional settings” button

- Choose user authentication

- And click on the “register client” button

- Connect with your user

# On the ISE server

## Add network devices

Configure the network device that serves as an intermediary between the endpoints and ISE.

Give a name, an IP address and the shared secret between the device and the RADIUS server.

Access:

Administration  Network Resources  Network Devices  Add

## Add identities

Create groups and users. Assign the user in the good group.

Access:

Administration  Identity Management  Groups

Administration  Identity Management  Identities

## Add policies

### Create conditions

Create the conditions, if the conditions are true, the results are applied.

Access:

Policy  Policy Sets  Create a new policy (click on ”+”)  Choose a name, click on “+” in the Conditions column  Conditions Studio

Choose a Dictionary  Choose a condition (e.g. Nas-Port-Type Wireless)  Save the condition

Virtual = SSH/Telnet/VTY

Async = Console/AUX serial

Ethernet = 802.1X

Wireless = Wi-Fi

Choose protocols:

Now choose the protocol(s) that ISE will use to communicate with the network device.

Access:

Allowed Protocols (click on ”+”)  define the necessary protocols  Choose the Allowed Protocol that you define  Save

Authentication:

Create the rule for the authentication.

Access:

In the column ”View” click on the right arrow. You will choose the authentication policy and the authorization policy.

Create a Rule for Authentication Policy  Condition Studio  Drag and drop the previous created condition

In the column ”Use”, choose who this Policy applies to (e.g. Internal Users).

Authorization:

Next, create an Authorization Policy. Choose the same condition that the Authentication Policy. Choose the group affected.

Access:

Conditions Studio  choose the icon ”Identity Group-Name”  ”equals”  choose the name of the desired group

In the column ”Results/Profiles” (click on ”+”) Create a ”New authorization profile” (click on ”+”)  Config the authorization policy (e.g. Assign a VLAN or a cisco av-pair)

Do not enter anything in the Security Groups column.

In the column Use choose ”Internal Users”  Choose the action to do if the authentication/authorization is KO or if the user can’t be found.

shell:priv-lvl=15

auth-proxy:priv-lvl=15

auth-proxy:proxyacl#1=permit icmp any any

auth-proxy:proxyacl#2=permit tcp any any